Proposed AC – DC block diagram.

For an ultra-high conversion efficiency (over 95%) and compact AC-DC power conversion board, we've thought about the following key aspects:

#### Functional Block:

- Input AC Voltage → EMI Filter → PFC Stage → Bridge Rectifier → LLC Resonant Converter.

System Architecture and Topology: Choosing the right topology is crucial for achieving high efficiency and compact size. For this application, a resonant converter topology, such as the LLC resonant converter, is ideal due to its high efficiency, soft-switching capabilities, and compact design. Additionally, for the DC-DC conversion stages, we will employ synchronous buck converters, known for their efficiency and compact size.

EMC/EMI Considerations: The design will include EMI filters and shielding to meet EMC standards for consumer electronics. A piezoelectric EMI filter will be considered to reduce conducted emissions.

Power Management and Control: A Power Management IC (PMIC) will be used for monitoring and controlling the output voltages and currents. This IC will include protection features such as over-voltage, over-current, and thermal shutdown.

Critical Components and Modules: Selection of components such as GaN-based transistors for high efficiency and compact size, transformers for isolation, and low-profile capacitors and inductors.

### Block Diagram High-Level Overview

### This is a draft overview of the components and their connections, ensuring a compact and efficient design suitable for consumer electronics applications. The choice of GaN transistors, resonant converter topology, and synchronous buck converters ensures high efficiency and compactness. The inclusion of EMC/EMI filtering and protection features ensures safety and compliance with regulatory standards.

### We expect that the supplier does through review... recommend any changes (if any) or recommend their system.

- Input Stage:

  - EMI Filter: Includes common-mode chokes and capacitors for noise suppression.

  - Bridge Rectifier: Converts AC to DC.

- Primary Stage (High Voltage Side):

  - LLC Resonant Converter: Provides efficient DC-DC conversion with galvanic isolation. Key components include resonant capacitors, a transformer, and synchronous rectifiers.

- Secondary Stage (Low Voltage Side):

  - Synchronous Buck Converters: For generating 3.3V, 5V, 12V, and 24V outputs. Each stage will have its own control circuitry for regulation.

- Control and Monitoring:

  - PMIC: Manages power conversion, monitors output voltages and currents, and provides protection features.

  - Current and Voltage Sensors: For monitoring and feedback.

#### Component and Functional Details

- Input Stage:

  - EMI Filter:

    - L1, C1, C2: Common-mode choke (L1 – Inductor Symbol) and Y-capacitors (C1,C2) for differential and common-mode noise suppression.

    - F1: Input fuse for overcurrent protection.

    - Components: Common-mode choke (L1), Y-capacitors (C1, C2), input fuse (F1). Review: TDK ACM7060-701-2P-TL00 , R46KI310050M1K, TDK B32922C3104M, Schurter 0001.2511

    - Connections:

      - AC input (90-260VAC, 50-60Hz) enters the EMI filter.

      - After the filter, the signal passes through the inrush current limiter (NTC thermistor).

  - Bridge Rectifier:

    - D1-D4: 4 Diodes arranged in bridge configuration for AC to DC conversion.

    - Components: Diodes (D1-D4). Review: STMicroelectronics STTH30R06CW,

    - Connections:

      - Output of the EMI filter feeds into the bridge rectifier.

      - The rectifier output is a DC voltage.

  - Inrush Current Limiter:

    - NTC Thermistor: To limit the inrush current.

- Primary Stage (High Voltage Side):

  - LLC Resonant Converter:

    - Q1, Q2: GaN FETs for high-efficiency switching (high-efficiency GaN transistors.)

    - Lr, Cr: Resonant inductor and capacitor in series.

    - T1: High-frequency transformer for isolation and voltage step-down.

    - Components: GaN FETs (Q1, Q2), resonant inductor (Lr), resonant capacitor (Cr), transformer (T1).

      - Review: EPC EPC2045, Infineon IGOT60R070D1, Texas Instruments LMG3410R070

      - Review: Coilcraft LPR6235 Series, SRR1260-100M

      - Review: b32674d3126k000

    - Connections:

      - Rectified DC input is fed into the LLC resonant converter.

      - The transformer provides galvanic isolation and voltage conversion.

  - Synchronous Rectification:

    - Q3, Q4: Synchronous rectifiers for efficient DC output.

    - Components: Synchronous rectifiers (Q3, Q4).

    - Connections:

      - The output of the transformer is rectified using synchronous rectifiers.

- Secondary Stage (Low Voltage Side):

  - Output Filters:

    - L2-L5, C3-C6: LC filter - Inductors and capacitors for smoothing the output voltages.

  - Synchronous Buck Converters:

    - For 3.3V, 5V, 12V, 24V outputs:

      - Q5-Q8: Synchronous rectifiers – transistors for each output stage.

      - L6-L9, C7-C10: Output filter components.

    - Components: Synchronous rectifiers (Q5-Q8), output inductors (L6-L9), output capacitors (C7-C10).

    - Outputs:

      - 3.3V, 4A: Synchronous buck converter 1.

      - 5V, 2A: Synchronous buck converter 2.

      - 12V, 2A: Synchronous buck converter 3.

      - 24V, 2A: Synchronous buck converter 4.

- Control and Monitoring:

  - PMIC: Integrated circuits for power management.

    - Functions: Voltage regulation, current monitoring, protection.

    - Connections:

      - Connected to all outputs for monitoring and regulation.

  - Sensors: For voltage and current feedback.

    - Components: Voltage and current sensors.

    - Connections:

      - Provide feedback to PMIC.

- Auxiliary Components:

  - Thermal Management: Heatsinks and thermal pads (GaN FETs, Synchronous Rectifiers, LLC, IC, Inductor)

  - Protection: TVS diodes, Zener diodes, (parallel with output lines for overvoltage protection.) and fuses for overcurrent protection.

    - Components: TVS diodes, Zener diodes, additional fuses.

    - Connections:

      - Placed across outputs and critical paths for protection.

### Annotations and Descriptions

- Input: 90-260VAC, 50-60Hz.

- High Voltage DC Output: 100VDC, 16-18A, 1.5kW.

- Low Voltage Outputs:

  - 3.3V, 4A.

  - 5V, 2A.

  - 12V, 2A.

  - 24V, 2A.

- Control Signals: PWM signals for controlling the converters, feedback signals for voltage and current monitoring.

Diagram Layout

- Left Side (Input Stage): EMI filter, bridge rectifier, and inrush current limiter.

- Center (Primary Stage): LLC resonant converter, transformer, and synchronous rectifiers.

- Right Side (Secondary Stage): Synchronous buck converters, each labeled with output voltages.

- Bottom (Control and Monitoring): PMIC, sensors.

- Top (Auxiliary): Thermal management and protection features.